#include <WiFiClientSecure.h>

#include <PubSubClient.h>

#include <ArduinoJson.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <stdbool.h>

#include "WiFi.h"

#include "driver/twai.h"

#define CAN_TX_PIN GPIO_NUM_21

#define CAN_RX_PIN GPIO_NUM_22

#define THINGNAME "vehicle_health"

#define I2C_SDA 18

#define I2C_SCL 19

const char WIFI_SSID[] = "Tej";

const char WIFI_PASSWORD[] = "123456789";

const char AWS_IOT_ENDPOINT[] = "aw6xkuuwtgkxj-ats.iot.ap-south-1.amazonaws.com";

uint8_t temperature;

//uint16_t average;

uint8_t Touch_Result;

uint16_t distance;

uint8_t x, y, z;

uint8_t flag;

uint8_t steering;

uint16_t vibration;

bool can_alive = false;

unsigned long lastCanRxTime = 0;

const unsigned long CAN_TIMEOUT = 3000;

LiquidCrystal_I2C lcd(0x27, 16, 2);

twai_general_config_t g_config = TWAI_GENERAL_CONFIG_DEFAULT(CAN_TX_PIN, CAN_RX_PIN, TWAI_MODE_NORMAL);

twai_timing_config_t t_config = TWAI_TIMING_CONFIG_500KBITS(); // must match STM32

twai_filter_config_t f_config = TWAI_FILTER_CONFIG_ACCEPT_ALL(); // receive all IDs

// Amazon Root CA 1

static const char AWS_CERT_CA[] PROGMEM = R"EOF(

-----BEGIN CERTIFICATE-----

MIIDQTCCAimgAwIBAgITBmyfz5m/jAo54vB4ikPmljZbyjANBgkqhkiG9w0BAQsF

ADA5MQswCQYDVQQGEwJVUzEPMA0GA1UEChMGQW1hem9uMRkwFwYDVQQDExBBbWF6

b24gUm9vdCBDQSAxMB4XDTE1MDUyNjAwMDAwMFoXDTM4MDExNzAwMDAwMFowOTEL

MAkGA1UEBhMCVVMxDzANBgNVBAoTBkFtYXpvbjEZMBcGA1UEAxMQQW1hem9uIFJv

b3QgQ0EgMTCCASIwDQYJKoZIhvcNAQEBBQADggEPADCCAQoCggEBALJ4gHHKeNXj

ca9HgFB0fW7Y14h29Jlo91ghYPl0hAEvrAIthtOgQ3pOsqTQNroBvo3bSMgHFzZM

9O6II8c+6zf1tRn4SWiw3te5djgdYZ6k/oI2peVKVuRF4fn9tBb6dNqcmzU5L/qw

IFAGbHrQgLKm+a/sRxmPUDgH3KKHOVj4utWp+UhnMJbulHheb4mjUcAwhmahRWa6

VOujw5H5SNz/0egwLX0tdHA114gk957EWW67c4cX8jJGKLhD+rcdqsq08p8kDi1L

93FcXmn/6pUCyziKrlA4b9v7LWIbxcceVOF34GfID5yHI9Y/QCB/IIDEgEw+OyQm

jgSubJrIqg0CAwEAAaNCMEAwDwYDVR0TAQH/BAUwAwEB/zAOBgNVHQ8BAf8EBAMC

AYYwHQYDVR0OBBYEFIQYzIU07LwMlJQuCFmcx7IQTgoIMA0GCSqGSIb3DQEBCwUA

A4IBAQCY8jdaQZChGsV2USggNiMOruYou6r4lK5IpDB/G/wkjUu0yKGX9rbxenDI

U5PMCCjjmCXPI6T53iHTfIUJrU6adTrCC2qJeHZERxhlbI1Bjjt/msv0tadQ1wUs

N+gDS63pYaACbvXy8MWy7Vu33PqUXHeeE6V/Uq2V8viTO96LXFvKWlJbYK8U90vv

o/ufQJVtMVT8QtPHRh8jrdkPSHCa2XV4cdFyQzR1bldZwgJcJmApzyMZFo6IQ6XU

5MsI+yMRQ+hDKXJioaldXgjUkK642M4UwtBV8ob2xJNDd2ZhwLnoQdeXeGADbkpy

rqXRfboQnoZsG4q5WTP468SQvvG5

-----END CERTIFICATE-----

)EOF";

// Device Certificate

static const char AWS_CERT_CRT[] PROGMEM = R"KEY(

-----BEGIN CERTIFICATE-----

MIIDWTCCAkGgAwIBAgIUXETAZoH6YBdFOJ1XiGWE8J9aVAgwDQYJKoZIhvcNAQEL

BQAwTTFLMEkGA1UECwxCQW1hem9uIFdlYiBTZXJ2aWNlcyBPPUFtYXpvbi5jb20g

SW5jLiBMPVNlYXR0bGUgU1Q9V2FzaGluZ3RvbiBDPVVTMB4XDTI2MDEyNDAzNDUw

MloXDTQ5MTIzMTIzNTk1OVowHjEcMBoGA1UEAwwTQVdTIElvVCBDZXJ0aWZpY2F0

ZTCCASIwDQYJKoZIhvcNAQEBBQADggEPADCCAQoCggEBAKe+EG714CqmWiyJlt6i

a98DfS7Xqjs5Uk13mvgdH967TN+11P3BO3GjuQNxGTa6lsy6hHeKwGHwNGD5ZlDd

LusdH66JRmQ+Q0DqqkxBA851yHut3JSGy9TScYiP6oVsthbgPU3p88Z+cI+kOsa9

wGm5ARHJP1UI/ASvFwl199PnfX9FaaAItxNZJLZid7l3yYtssQQ1m/fm3s4cb19J

wn0LjA6A0ApjcAN5inAk4ocQwwMu1Q+m2gjkEeTDFgels/YaX+35i9Paq87pzrno

Qv6n1FJGOVa67IqBxQoE+qMyh0kouA+8Ji845pAdpl1raMCWnasVULCHGkvAL80Q

STMCAwEAAaNgMF4wHwYDVR0jBBgwFoAUk7k+10HXikf6MVi3wXhw/1XuINMwHQYD

VR0OBBYEFNo87ilGCJj7XscKGia5kF3dIpv4MAwGA1UdEwEB/wQCMAAwDgYDVR0P

AQH/BAQDAgeAMA0GCSqGSIb3DQEBCwUAA4IBAQBvaQ5pQJt+3+DFxeU4s2w7TrZ9

ZBJSGIzFDNyptdpO1IHFLwOmLPDv8EG32LlDDXwOCLn7DeRsVLt8HXmeq1HoLMBj

MXM+pR5cIfDi686xrjlEHRVsDQE3647aKBUT397Rch8+5KXBcjohDqA6UFMFqg7s

N6C6dkFXkl3P73jv7Yw5Owy8/uk0LjeKnjnUkakUsQYOIxFwk0/Nv4IQmj6B6hnD

3te946CvgXSYV9EDervyx0yB/hF3sVyt4uW5yHvLqmPsTFFubDXA/gPR+IkgxvB5

vCYHn7+HGoAbGcDqgigFGkRZe7BGxDLsANuYCC2W48Dnstf0Abdhzq9okW5H

-----END CERTIFICATE-----

)KEY";

// Device Private Key

static const char AWS_CERT_PRIVATE[] PROGMEM = R"KEY(

-----BEGIN RSA PRIVATE KEY-----

MIIEpQIBAAKCAQEAp74QbvXgKqZaLImW3qJr3wN9LteqOzlSTXea+B0f3rtM37XU

/cE7caO5A3EZNrqWzLqEd4rAYfA0YPlmUN0u6x0frolGZD5DQOqqTEEDznXIe63c

lIbL1NJxiI/qhWy2FuA9Tenzxn5wj6Q6xr3AabkBEck/VQj8BK8XCXX30+d9f0Vp

oAi3E1kktmJ3uXfJi2yxBDWb9+bezhxvX0nCfQuMDoDQCmNwA3mKcCTihxDDAy7V

D6baCOQR5MMWB6Wz9hpf7fmL09qrzunOuehC/qfUUkY5VrrsioHFCgT6ozKHSSi4

D7wmLzjmkB2mXWtowJadqxVQsIcaS8AvzRBJMwIDAQABAoIBAQCNHQK8JhJwLsP7

ZcBLCP6QfWHpbzrhWcaYLv9qrS2rdYwlQtWsp8t3PVGeyJ16ghvsnRhlbI+NlMKr

tgPSEVCwMC7/6LIUEKEJUqDzR1qJbGK9UTUYqhF9t5AVX2Te2+aPdhN8oU0X9jSB

UpqzqNve1OlGajPiOI2y7w0FQ326gE22SD4wBxsSSbegCkZ7CE9SpI8B/M1VFtOp

a10dNGn0DuM/g5Ihjg+cs9SplywrrWqgWe6MGcFZuUQk0ODhLc+nkJ5QHB6FAlB+

E3LymqZhNNIbh7tWxk/m2qQaEeE6XpYx7x6y4n7hac9FRnGDsuIsZAMh9yrmdQ8w

7SYnmv1RAoGBANv086gR8zI1c6OvLeVidqlDIHoQinN6Gfj1m1eKh23SJIbLroWn

9UAg2El986cgrVh+tbNcRN6RDWCJtWVsm2KSv7W23hD2UjE9Fkk/fCJT2YSL0/po

iXPa+XeU+3TyIdkgh0Yn6nEegHnpNbYlNu9M/+UgCTPIT9+R8FabxHdpAoGBAMM6

wG6Xc/HujGT/YI9csFSrJSuuEh9LIuvMHHOMQrL6/uiz59eDTLB7qFs06uQ7Tg9f

+C67/qQdpgzOMGZrSQR0tHjMNici8ANdVXW6U1inAIlz7Q2JgSlalJbH+LipAoZc

8ggK46VAJ0CpKfsyGOAI+QIX6QT0Ntmr2Q5ETiQ7AoGBALILWIPOa9ZTkBSKA5Ov

HTArxwknhHqu9oEghirZVoCd3rVCADXuW8YaEBmFG4pIGCkPABN8d77VGUfwkGXW

AP1MnmP/70c5rBRxRy3pdpju2J7C9Ma4gbLHKLIUZWsnaCCyhO1T+CCyuPp1vOWA

wDP6VxTn1qsU1Rq6rhndIq4pAoGAN/cIoOtTtn/eyTkYEiDM/4iMQEhHM6dTXGsH

oOwNVsdy+XxRMsuV2WWpbnPt/f0uNGQbfjIwHhXNuxAEeee5Z8iYz6QixfdS/HPu

ArCmIdzF36FWzDGl0BgCTnMGztDMQ34zd7C/b2jDl9qmiTbkQJzx3PLDvsLPUnjb

4SYdjG8CgYEA2AsbeA6HMPd88n7dJ5Fhn800MUpk6l3ed7XNMqh3VVGdDZZWQAPl

GYOFVmn0zIUA31PxbJT3UGeBIE2QqqiasaTXqmWNzRcpMDarPV19H/vIhEQizL09

1DlK3Tq5Z26CAbcONMSctLPUoXUc3WYBZWjtLYMwCW2xhFEukYXpI1w=

-----END RSA PRIVATE KEY-----

)KEY";

#define AWS_IOT_PUBLISH_TOPIC   "vehicle_health_pub"

#define AWS_IOT_SUBSCRIBE_TOPIC "vehicle_health_sub"

WiFiClientSecure net = WiFiClientSecure();

PubSubClient client(net);

void publishMessage();

//void messageHandler();

void connectAWS();

void setup() {

Serial.begin(115200);

Wire.begin(I2C_SDA, I2C_SCL);

lcd.init();                      // initialize the lcd

lcd.init();

// Print a message to the LCD.

lcd.backlight();

lcd.setCursor(0, 0);

lcd.setCursor(0,1);

connectAWS();

lcd.print("AWS CONNECTED");

lcd.clear();

// Install CAN driver

if (twai_driver_install(&g_config, &t_config, &f_config) != ESP_OK) {

Serial.println("Failed to install TWAI driver");

while (1);

}

// Start CAN driver

if (twai_start() != ESP_OK) {

Serial.println("Failed to start TWAI driver");

while (1);

}

Serial.println("ESP32 CAN Receiver Ready");

}

void loop() {

twai_message_t rx_frame;

if (twai_receive(&rx_frame, pdMS_TO_TICKS(100)) == ESP_OK  && rx_frame.identifier == 0x123)

{

can_alive = true;

lastCanRxTime = millis();

temperature = rx_frame.data[0];

distance = rx_frame.data[2];

x = rx_frame.data[3];

y = rx_frame.data[4];

z = rx_frame.data[5];

vibration = abs(sqrt(x*x + y*y + z*z) - 350);

Touch_Result = rx_frame.data[6];

Serial.print("temp: ");

Serial.println(temperature);

Serial.print("distance: ");

Serial.println(distance);

Serial.print("vibration: ");

Serial.println(vibration);

Serial.print("Touch_Result: ");

Serial.println(Touch_Result);

Serial.println("data recieved: ");

lcd.setCursor(0, 0);

lcd.print("T:");

lcd.print(temperature);

lcd.print("  D:");

lcd.print(distance);

lcd.print("   ");  // clear leftovers

lcd.setCursor(0, 1);

lcd.print("V:");

lcd.print(vibration);

lcd.print("  Touch:");

lcd.print(Touch_Result);

lcd.print(" ");

if (!client.connected())

{

connectAWS();

}

client.loop();

static unsigned long lastMillis = 0;

if (millis() - lastMillis > 5000)

{

lastMillis = millis();

publishMessage();

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Sending to AWS");

}

}

if (millis() - lastCanRxTime > CAN_TIMEOUT) {

can_alive = false;

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("CAN DATA LOST");

lcd.setCursor(0, 1);

lcd.print("Check STM32");

Serial.println("CAN DATA LOST");

}

delay(1000);

}

void publishMessage() {

StaticJsonDocument<200> doc;

doc["temperature"] = temperature;

doc["oil"] = distance;

doc["vibration"]  = vibration;

char jsonBuffer[512];

serializeJson(doc, jsonBuffer);

client.publish(AWS_IOT_PUBLISH_TOPIC, jsonBuffer);

}

void connectAWS() {

WiFi.mode(WIFI_STA);

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

Serial.println("Connecting to Wi-Fi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

lcd.setCursor(0, 0);

lcd.print("Connecting WiFi");

delay(100);

}

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("WiFi connected!");

// Configure WiFiClientSecure with AWS IoT certificates

net.setCACert(AWS_CERT_CA);

net.setCertificate(AWS_CERT_CRT);

net.setPrivateKey(AWS_CERT_PRIVATE);

client.setServer(AWS_IOT_ENDPOINT, 8883);

//client.setCallback(messageHandler);

Serial.println("Connecting to AWS IOT");

while (!client.connect(THINGNAME)) {

Serial.print(".");

delay(100);

}

lcd.setCursor(0, 0);

lcd.print("Connected to AWS");

if (!client.connected())

{

Serial.println("AWS IoT Timeout!");

lcd.setCursor(0, 0);

lcd.print("TIMEOUT AWS");

return;

}

client.subscribe(AWS_IOT_SUBSCRIBE_TOPIC);

Serial.println("AWS IoT Connected!");

}

void messageHandler(char* topic, byte* payload, unsigned int length) {

Serial.print("Message arrived [");

Serial.print(topic);

Serial.print("]: ");

for (int i = 0; i < length; i++) {

Serial.print((char)payload[i]);

}

Serial.println();

}