# Tab 1

#include "driver/twai.h"

#define CAN_TX_PIN GPIO_NUM_21

#define CAN_RX_PIN GPIO_NUM_22

void setup() {

Serial.begin(115200);

delay(1000);

// Configure TWAI (CAN) driver

twai_general_config_t g_config = TWAI_GENERAL_CONFIG_DEFAULT(CAN_TX_PIN, CAN_RX_PIN, TWAI_MODE_NORMAL);

twai_timing_config_t t_config = TWAI_TIMING_CONFIG_500KBITS(); // must match STM32

twai_filter_config_t f_config = TWAI_FILTER_CONFIG_ACCEPT_ALL(); // receive all IDs

// Install CAN driver

if (twai_driver_install(&g_config, &t_config, &f_config) != ESP_OK) {

Serial.println("Failed to install TWAI driver");

while (1);

}

// Start CAN driver

if (twai_start() != ESP_OK) {

Serial.println("Failed to start TWAI driver");

while (1);

}

Serial.println("ESP32 CAN Receiver Ready");

}

void loop() {

twai_message_t rx_frame;

if (twai_receive(&rx_frame, pdMS_TO_TICKS(100)) == ESP_OK) {

if (rx_frame.identifier == 0x123) {

uint8_t value = rx_frame.data[0];

Serial.print("Received value: ");

Serial.println(value);

}

}

delay(300);

}

# Tab 2

/* USER CODE BEGIN Header */

/**

******************************************************************************

* @file           : main.c

* @brief          : Main program body

******************************************************************************

* @attention

*

* Copyright (c) 2026 STMicroelectronics.

* All rights reserved.

*

* This software is licensed under terms that can be found in the LICENSE file

* in the root directory of this software component.

* If no LICENSE file comes with this software, it is provided AS-IS.

*

******************************************************************************

*/

/* USER CODE END Header */

/* Includes ------------------------------------------------------------------*/

#include "main.h"

/* Private includes ----------------------------------------------------------*/

/* USER CODE BEGIN Includes */

/* USER CODE END Includes */

/* Private typedef -----------------------------------------------------------*/

/* USER CODE BEGIN PTD */

/* USER CODE END PTD */

/* Private define ------------------------------------------------------------*/

/* USER CODE BEGIN PD */

/* USER CODE END PD */

/* Private macro -------------------------------------------------------------*/

/* USER CODE BEGIN PM */

/* USER CODE END PM */

/* Private variables ---------------------------------------------------------*/

CAN_HandleTypeDef hcan1;

CAN_HandleTypeDef hcan2;

/* USER CODE BEGIN PV */

CAN_TxHeaderTypeDef TxHeader;

uint8_t TxData[8];

uint32_t TxMailbox;

/* USER CODE END PV */

/* Private function prototypes -----------------------------------------------*/

void SystemClock_Config(void);

static void MX_GPIO_Init(void);

static void MX_CAN2_Init(void);

static void MX_CAN1_Init(void);

/* USER CODE BEGIN PFP */

/* USER CODE END PFP */

/* Private user code ---------------------------------------------------------*/

/* USER CODE BEGIN 0 */

/* USER CODE END 0 */

/**

* @brief  The application entry point.

* @retval int

*/

int main(void)

{

/* USER CODE BEGIN 1 */

/* USER CODE END 1 */

/* MCU Configuration--------------------------------------------------------*/

/* Reset of all peripherals, Initializes the Flash interface and the Systick. */

HAL_Init();

/* USER CODE BEGIN Init */

/* USER CODE END Init */

/* Configure the system clock */

SystemClock_Config();

/* USER CODE BEGIN SysInit */

/* USER CODE END SysInit */

/* Initialize all configured peripherals */

MX_GPIO_Init();

MX_CAN2_Init();

MX_CAN1_Init();

/* USER CODE BEGIN 2 */

CAN_FilterTypeDef canfilter;

canfilter.FilterBank = 14;              // CAN2 uses bank >=14

canfilter.FilterMode = CAN_FILTERMODE_IDMASK;

canfilter.FilterScale = CAN_FILTERSCALE_32BIT;

canfilter.FilterIdHigh = 0x0000;

canfilter.FilterIdLow  = 0x0000;

canfilter.FilterMaskIdHigh = 0x0000;

canfilter.FilterMaskIdLow  = 0x0000;

canfilter.FilterFIFOAssignment = CAN_RX_FIFO0;

canfilter.FilterActivation = ENABLE;

canfilter.SlaveStartFilterBank = 14;

HAL_CAN_ConfigFilter(&hcan2, &canfilter);

HAL_CAN_Start(&hcan2);

TxHeader.StdId = 0x123;

TxHeader.IDE = CAN_ID_STD;

TxHeader.RTR = CAN_RTR_DATA;

TxHeader.DLC = 1; // 1 byte data

TxHeader.TransmitGlobalTime = DISABLE;

/* USER CODE END 2 */

/* Infinite loop */

/* USER CODE BEGIN WHILE */

while (1)

{

for(uint8_t value = 1; value <= 100; value++)

{

TxData[0] = value; // send 1 byte

// Wait for free mailbox and send

while(HAL_CAN_GetTxMailboxesFreeLevel(&hcan2) == 0);

HAL_CAN_AddTxMessage(&hcan2, &TxHeader, TxData, &TxMailbox);

HAL_GPIO_TogglePin(GPIOD, GPIO_PIN_15);

HAL_Delay(200); // small delay

/* USER CODE END WHILE */

/* USER CODE BEGIN 3 */

}

/* USER CODE END 3 */

}

}

/**

* @brief System Clock Configuration

* @retval None

*/

void SystemClock_Config(void)

{

RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

/** Configure the main internal regulator output voltage

*/

__HAL_RCC_PWR_CLK_ENABLE();

__HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE1);

/** Initializes the RCC Oscillators according to the specified parameters

* in the RCC_OscInitTypeDef structure.

*/

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSI;

RCC_OscInitStruct.HSIState = RCC_HSI_ON;

RCC_OscInitStruct.HSICalibrationValue = RCC_HSICALIBRATION_DEFAULT;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_ON;

RCC_OscInitStruct.PLL.PLLSource = RCC_PLLSOURCE_HSI;

RCC_OscInitStruct.PLL.PLLM = 8;

RCC_OscInitStruct.PLL.PLLN = 84;

RCC_OscInitStruct.PLL.PLLP = RCC_PLLP_DIV2;

RCC_OscInitStruct.PLL.PLLQ = 4;

if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK)

{

Error_Handler();

}

/** Initializes the CPU, AHB and APB buses clocks

*/

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK|RCC_CLOCKTYPE_SYSCLK

|RCC_CLOCKTYPE_PCLK1|RCC_CLOCKTYPE_PCLK2;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_PLLCLK;

RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV2;

RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV1;

if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_2) != HAL_OK)

{

Error_Handler();

}

}

/**

* @brief CAN1 Initialization Function

* @param None

* @retval None

*/

static void MX_CAN1_Init(void)

{

/* USER CODE BEGIN CAN1_Init 0 */

/* USER CODE END CAN1_Init 0 */

/* USER CODE BEGIN CAN1_Init 1 */

/* USER CODE END CAN1_Init 1 */

hcan1.Instance = CAN1;

hcan1.Init.Prescaler = 16;

hcan1.Init.Mode = CAN_MODE_NORMAL;

hcan1.Init.SyncJumpWidth = CAN_SJW_1TQ;

hcan1.Init.TimeSeg1 = CAN_BS1_1TQ;

hcan1.Init.TimeSeg2 = CAN_BS2_1TQ;

hcan1.Init.TimeTriggeredMode = DISABLE;

hcan1.Init.AutoBusOff = DISABLE;

hcan1.Init.AutoWakeUp = DISABLE;

hcan1.Init.AutoRetransmission = DISABLE;

hcan1.Init.ReceiveFifoLocked = DISABLE;

hcan1.Init.TransmitFifoPriority = DISABLE;

if (HAL_CAN_Init(&hcan1) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN CAN1_Init 2 */

/* USER CODE END CAN1_Init 2 */

}

/**

* @brief CAN2 Initialization Function

* @param None

* @retval None

*/

static void MX_CAN2_Init(void)

{

/* USER CODE BEGIN CAN2_Init 0 */

/* USER CODE END CAN2_Init 0 */

/* USER CODE BEGIN CAN2_Init 1 */

/* USER CODE END CAN2_Init 1 */

hcan2.Instance = CAN2;

hcan2.Init.Prescaler = 6;

hcan2.Init.Mode = CAN_MODE_NORMAL;

hcan2.Init.SyncJumpWidth = CAN_SJW_1TQ;

hcan2.Init.TimeSeg1 = CAN_BS1_11TQ;

hcan2.Init.TimeSeg2 = CAN_BS2_2TQ;

hcan2.Init.TimeTriggeredMode = DISABLE;

hcan2.Init.AutoBusOff = DISABLE;

hcan2.Init.AutoWakeUp = DISABLE;

hcan2.Init.AutoRetransmission = DISABLE;

hcan2.Init.ReceiveFifoLocked = DISABLE;

hcan2.Init.TransmitFifoPriority = DISABLE;

if (HAL_CAN_Init(&hcan2) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN CAN2_Init 2 */

/* USER CODE END CAN2_Init 2 */

}

/**

* @brief GPIO Initialization Function

* @param None

* @retval None

*/

static void MX_GPIO_Init(void)

{

GPIO_InitTypeDef GPIO_InitStruct = {0};

/* USER CODE BEGIN MX_GPIO_Init_1 */

/* USER CODE END MX_GPIO_Init_1 */

/* GPIO Ports Clock Enable */

__HAL_RCC_GPIOB_CLK_ENABLE();

__HAL_RCC_GPIOD_CLK_ENABLE();

__HAL_RCC_GPIOA_CLK_ENABLE();

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_12|GPIO_PIN_13|GPIO_PIN_14|GPIO_PIN_15, GPIO_PIN_RESET);

/*Configure GPIO pins : PD12 PD13 PD14 PD15 */

GPIO_InitStruct.Pin = GPIO_PIN_12|GPIO_PIN_13|GPIO_PIN_14|GPIO_PIN_15;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOD, &GPIO_InitStruct);

/* USER CODE BEGIN MX_GPIO_Init_2 */

/* USER CODE END MX_GPIO_Init_2 */

}

/* USER CODE BEGIN 4 */

/* USER CODE END 4 */

/**

* @brief  This function is executed in case of error occurrence.

* @retval None

*/

void Error_Handler(void)

{

/* USER CODE BEGIN Error_Handler_Debug */

/* User can add his own implementation to report the HAL error return state */

__disable_irq();

while (1)

{

}

/* USER CODE END Error_Handler_Debug */

}

#ifdef  USE_FULL_ASSERT

/**

* @brief  Reports the name of the source file and the source line number

*         where the assert_param error has occurred.

* @param  file: pointer to the source file name

* @param  line: assert_param error line source number

* @retval None

*/

void assert_failed(uint8_t *file, uint32_t line)

{

/* USER CODE BEGIN 6 */

/* User can add his own implementation to report the file name and line number,

ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */

/* USER CODE END 6 */

}

#endif /* USE_FULL_ASSERT */

# Tab 3