/* USER CODE BEGIN Header */

/**

******************************************************************************

* @file           : main.c

* @brief          : FreeRTOS Sensor + Ultrasonic + CAN

******************************************************************************

*/

#include "main.h"

#include "adc.h"

#include "can.h"

#include "i2c.h"

#include "tim.h"

#include "gpio.h"

/* USER CODE BEGIN Includes */

#include "FreeRTOS.h"

#include "task.h"

#include "semphr.h"

#include "SEGGER_SYSVIEW.h"

#include "Disc_F407.h"

/* USER CODE END Includes */

/* USER CODE BEGIN PD */

#define SAMPLE 100

#define TRIG_PORT GPIOA

#define TRIG_PIN GPIO_PIN_1

#define ECHO_PORT GPIOA

#define ECHO_PIN GPIO_PIN_0

#define ADXL_ADDR (0x53 << 1)

#define STACK_SIZE 768

#define TOUCH1_PORT GPIOD

#define TOUCH1_PIN  GPIO_PIN_2

#define TOUCH2_PORT GPIOD

#define TOUCH2_PIN  GPIO_PIN_6

#define BUZZER_PORT GPIOE

#define BUZZER_PIN  GPIO_PIN_3

/* USER CODE END PD */

/* USER CODE BEGIN PV */

SemaphoreHandle_t sensor_sem;

volatile float temp;

volatile float avg;

uint8_t rec_data[6];

int16_t x, y, z;

uint32_t distance_cm = 0;

uint8_t TxData[7];

volatile uint8_t touch1_state = 0;

volatile uint8_t touch2_state = 0;

volatile uint8_t touch_status = 0;

uint32_t TxMailbox;

CAN_TxHeaderTypeDef TxHeader;

/* USER CODE END PV */

/* USER CODE BEGIN PFP */

float temperature(void);

void vibration(void);

uint32_t ultrasonic_read_cm(void);

void data_collection(void *argument);

void data_send_to_esp(void *argument);

void ADXL_Init(I2C_HandleTypeDef *hi2c);

void ADXL_Read(I2C_HandleTypeDef *hi2c);

void CAN_Init(void);

void SystemClock_Config(void);

void Error_Handler(void);

void read_touch_sensors(void);

void buzzer_control(void);

/* USER CODE END PFP */

int main(void)

{

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

MX_ADC3_Init();

MX_I2C1_Init();

MX_CAN1_Init();

MX_CAN2_Init();

MX_TIM2_Init();

HAL_TIM_Base_Start(&htim2);

SEGGER_SYSVIEW_Conf();

sensor_sem = xSemaphoreCreateBinary();

xSemaphoreGive(sensor_sem);

CAN_Init();

ADXL_Init(&hi2c1);

xTaskCreate(data_collection, "SensorTask", STACK_SIZE, NULL, tskIDLE_PRIORITY + 2, NULL);

xTaskCreate(data_send_to_esp, "CANTask", STACK_SIZE, NULL, tskIDLE_PRIORITY + 1, NULL);

vTaskStartScheduler();

while (1) {}

}

/* ================= SENSOR TASK ================= */

void data_collection(void *argument)

{

while (1)

{

if (xSemaphoreTake(sensor_sem, portMAX_DELAY) == pdPASS)

{

SEGGER_SYSVIEW_PrintfHost("DATA_COLLECTION_TASK_RUNNING");

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_14, GPIO_PIN_SET);

temp = temperature();

vibration();

distance_cm = ultrasonic_read_cm();

read_touch_sensors();

buzzer_control();

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_14, GPIO_PIN_RESET);

xSemaphoreGive(sensor_sem);

}

vTaskDelay(pdMS_TO_TICKS(400));

}

}

/* ================= TEMPERATURE ================= */

float temperature(void)

{

uint32_t sum = 0;

for (int i = 0; i < SAMPLE; i++)

{

HAL_ADC_Start(&hadc3);

HAL_ADC_PollForConversion(&hadc3, 10);

sum += HAL_ADC_GetValue(&hadc3);

HAL_ADC_Stop(&hadc3);

}

avg = (float)sum / SAMPLE;

return (avg * 3.3f * 100.0f) / 4095.0f;

}

/* ================= ADXL ================= */

void vibration(void)

{

ADXL_Read(&hi2c1);

}

void ADXL_Init(I2C_HandleTypeDef *hi2c)

{

uint8_t data;

HAL_I2C_Mem_Read(hi2c, ADXL_ADDR, 0x00, 1, &data, 1, 100);

if (data != 0xE5) return;

data = 0x0B;

HAL_I2C_Mem_Write(hi2c, ADXL_ADDR, 0x31, 1, &data, 1, 100);

data = 0x08;

HAL_I2C_Mem_Write(hi2c, ADXL_ADDR, 0x2D, 1, &data, 1, 100);

}

void ADXL_Read(I2C_HandleTypeDef *hi2c)

{

if (HAL_I2C_Mem_Read(hi2c, ADXL_ADDR, 0x32, 1, rec_data, 6, 10) != HAL_OK)

return;

x = (int16_t)((rec_data[1] << 8) | rec_data[0]);

y = (int16_t)((rec_data[3] << 8) | rec_data[2]);

z = (int16_t)((rec_data[5] << 8) | rec_data[4]);

}

/* ================= ULTRASONIC ================= */

uint32_t ultrasonic_read_cm(void)

{

uint32_t timeout = 30000;

HAL_GPIO_WritePin(TRIG_PORT, TRIG_PIN, GPIO_PIN_RESET);

vTaskDelay(pdMS_TO_TICKS(2));

HAL_GPIO_WritePin(TRIG_PORT, TRIG_PIN, GPIO_PIN_SET);

__HAL_TIM_SET_COUNTER(&htim2, 0);

while (__HAL_TIM_GET_COUNTER(&htim2) < 10);

HAL_GPIO_WritePin(TRIG_PORT, TRIG_PIN, GPIO_PIN_RESET);

__HAL_TIM_SET_COUNTER(&htim2, 0);

while (HAL_GPIO_ReadPin(ECHO_PORT, ECHO_PIN) == GPIO_PIN_RESET)

if (__HAL_TIM_GET_COUNTER(&htim2) > timeout) return 0;

uint32_t start = __HAL_TIM_GET_COUNTER(&htim2);

while (HAL_GPIO_ReadPin(ECHO_PORT, ECHO_PIN) == GPIO_PIN_SET)

if (__HAL_TIM_GET_COUNTER(&htim2) > timeout) return 0;

uint32_t stop = __HAL_TIM_GET_COUNTER(&htim2);

return (stop - start) / 58;

}

/* ================= CAN TASK ================= */

void data_send_to_esp(void *argument)

{

while (1)

{

if (xSemaphoreTake(sensor_sem, portMAX_DELAY) == pdPASS)

{

SEGGER_SYSVIEW_PrintfHost("CAN_DATA_SEND_TO_ESP32_TASK_RUNNING");

TxData[0] = (uint8_t)temp;

TxData[1] = (distance_cm >> 8) & 0xFF;

TxData[2] = distance_cm & 0xFF;

TxData[3] = x & 0xFF;

TxData[4] = y & 0xFF;

TxData[5] = z & 0xFF;

TxData[6] = touch_status;

if (HAL_CAN_GetTxMailboxesFreeLevel(&hcan2) > 0)

HAL_CAN_AddTxMessage(&hcan2, &TxHeader, TxData, &TxMailbox);

xSemaphoreGive(sensor_sem);

}

vTaskDelay(pdMS_TO_TICKS(500));

}

}

/* ================= CAN INIT ================= */

void CAN_Init(void)

{

CAN_FilterTypeDef filter = {0};

filter.FilterBank = 14;

filter.FilterMode = CAN_FILTERMODE_IDMASK;

filter.FilterScale = CAN_FILTERSCALE_32BIT;

filter.FilterFIFOAssignment = CAN_FILTER_FIFO0;

filter.FilterActivation = ENABLE;

HAL_CAN_ConfigFilter(&hcan2, &filter);

TxHeader.StdId = 0x123;

TxHeader.IDE = CAN_ID_STD;

TxHeader.RTR = CAN_RTR_DATA;

TxHeader.DLC = 8;

HAL_CAN_Start(&hcan2);

}

/* ================= SYSTEM CLOCK ================= */

void SystemClock_Config(void)

{

RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

__HAL_RCC_PWR_CLK_ENABLE();

__HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE1);

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSE;

RCC_OscInitStruct.HSEState = RCC_HSE_ON;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_ON;

RCC_OscInitStruct.PLL.PLLSource = RCC_PLLSOURCE_HSE;

RCC_OscInitStruct.PLL.PLLM = 8;

RCC_OscInitStruct.PLL.PLLN = 336;

RCC_OscInitStruct.PLL.PLLP = RCC_PLLP_DIV2;

RCC_OscInitStruct.PLL.PLLQ = 7;

if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK)

Error_Handler();

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_SYSCLK |

RCC_CLOCKTYPE_PCLK1 |

RCC_CLOCKTYPE_PCLK2;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_PLLCLK;

RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV4;

RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV2;

if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_5) != HAL_OK)

Error_Handler();

}

/* ================= ERROR HANDLER ================= */

void Error_Handler(void)

{

__disable_irq();

while (1)

{

HAL_GPIO_TogglePin(GPIOD, GPIO_PIN_14);

HAL_Delay(200);

}

}

void read_touch_sensors(void)

{

touch_status = 0;

if (HAL_GPIO_ReadPin(TOUCH1_PORT, TOUCH1_PIN) == GPIO_PIN_SET)

touch_status |= (1 << 0);   // PD2 → bit0

if (HAL_GPIO_ReadPin(TOUCH2_PORT, TOUCH2_PIN) == GPIO_PIN_SET)

touch_status |= (1 << 0);   // PD6 → bit1

touch1_state = touch_status & 0x01;

touch2_state = (touch_status >> 1) & 0x01;

}

void buzzer_control(void)

{

if ((touch1_state == 0) && (touch2_state == 0))

{

HAL_GPIO_WritePin(BUZZER_PORT, BUZZER_PIN, GPIO_PIN_SET);   // BUZZ

}

else

{

HAL_GPIO_WritePin(BUZZER_PORT, BUZZER_PIN, GPIO_PIN_RESET); // SILENT

}

}